Min-Cost Max-Flow Interception Assignment Integration and Validation Report

Yeqi Sang

1. Introduction

Version 46 (V46) replaces the interceptor selection module in the original simulation with a global assignment strategy based on min-cost max-flow (MCMF). To validate this new module in a reliable and interpretable way, the validation process was divided into four stages.  s

I separated the assignment algorithm into its own module. First, I tested the algorithm logic by itself using mock sensor data, target data, and bidding costs. Then, I tested whether the same algorithm could work with the current simulation structure.

For the simulation-level test, I used event injection for a short simulation period. This means I set simulation in a short time (8 sec and 30 sec), manually created a specific situation where two targets enter PENDING_SELECTION at the same time. This lets me check whether the system can choose the best overall interceptor combination for both targets globally, instead of selecting interceptors for each target separately.

2. What Was Designed in V46

V46 Design Overview

In V45, interceptor selection was performed inside the simulation through a min-max conflict-resolution strategy based on bidding logic. In V46, a clearer strategy boundary was introduced so that the assignment module could be tested independently from the rest of the simulation.

The main components introduced in V46 are as follows.

2.1 MCMF Solver

The new interceptor assignment strategy is formulated as a min-cost max-flow problem. Each calling target is expanded into interceptor slots, where the number of required slots is equal to the number of calling targets multiplied by the maximum number of active interceptors per target. Each candidate sensor has capacity 1, meaning that one sensor cannot be assigned to more than one target at the same time.

Table 1 example: we have targets t1 and t2. Available sensors are [ 5, 6, 7, 8, 9]. And for each target, the cost matrix (bidding cost) is:

| Target | Sensor 5 | Sensor 6 | Sensor 7 | Sensor 8 | Sensor 9 |
| --- | --- | --- | --- | --- | --- |
| t1 | 1 | 2 | 3 | 4 | 5 |
| t2 | 1 | 2 | 1000 | 1000 | 1000 |

Table 1: cost matrix for Section 2.1 example

Next, the algorithm creates interceptor slots. Since for each target, we want to assign them two interceptors, so each target has 2 interceptor slots, 4 interceptor slots in total: t1-slot1, t1-slot2, t2-slot1, t2-slot2

Figure 1 provides a brief overview of the flow network.

Each sensor is represented by a corresponding sensor node. The network also includes a source node and a sink node, which define the start and end points of each flow path.

Figure 1. Initial flow network: Each sensor node has directed edges to each interceptor slot

For edges:

We have source → sensor node, and capacity=1 represents each sensor candidate can be at most used once:

e.g.	source → s5, capacity = 1, cost = 0

source → s6, capacity = 1, cost = 0

….

source → s9, capacity = 1, cost = 0

Then we have sensor → interceptor slot.

s5 → t1-slot1, cost = 1

s5 → t2-slot1, cost = 1

….

s7 → t2-slot1, cost = 1000

s7 → t2-slot 2, cost = 1000

….

s9 → t2-slot 2, cost = 1000

The cost is based on the Table 1.

We have interceptor slot → sink, capacity=1 represents each interceptor slot can only accept one sensor candidate.

t1-slot1 → sink, capacity = 1, cost = 0

…

t2-slot2 → sink, capacity = 1, cost = 0

Then, the algorithm tries to find the maximum number of flows with the minimum total cost. In this model, one unit of flow represents one sensor assignment. Each flow starts from the source node, passes through one sensor and one interceptor slot, and ends at the sink node.

Figure 2.  reverse edge t1_slot1 → s5 reduces the global cost

For example, in Figure 2, the algorithm may first create this flow:

source →s5→t1_slot1→sink

This means that sensor 5 is assigned to t1_slot1. After this flow is created, the residual network includes a reverse edge:

t1_slot1→s5

This reverse edge allows the algorithm to adjust the previous assignment later if a better global solution is found.

Later, suppose the algorithm considers assigning sensor 7 directly to t2_slot1:

source →s7→t2_slot1→sink

This assignment has a very high cost because sensor 7 is not a good choice for target 2, with cost 1000. However, because the reverse edge t1_slot1 → s5 exists, the algorithm can re-route the assignment:

source →s7→t1_slot1→s5→t2_slot1→sink

This path means that sensor 7 replaces sensor 5 for t1_slot1, with cost 1, as described in Table, and sensor 5 is moved to t2_slot1, with cost 3. In other words, the algorithm swaps the previous assignment to reduce the total cost. This is how the residual edges allow MCMF to improve the global assignment instead of keeping the first local choice fixed.

2.2 Validation with Mock Data

A separate mock test file is created to test the MCMF algorithm outside the full simulation. The mock data includes sensor states, target states and mocked interception points, which have the same structure as the real simulation data. This test checks whether the MCMF solver can correctly use the simulation-style inputs and be integrated into the existing simulation system.

2.3 A/B Switching Between MCMF and Min-max for Validation

The wrapper function solveInterceptorAssignments(...) preserves the original V45 min-max assignment strategy while enabling the new MCMF solver. This provides a fair A/B comparison framework, because both algorithms can be evaluated using the same candidate sensors and the same bidding cost matrix.

The MCMF objective is two-step: it first maximizes the number of assigned interceptor slots, and then minimizes the total assignment cost among solutions with the same number of assigned slots.

3. Validation Strategy and Results

3.1 Stage 1: Unit Test

Stage 1 focuses on validating the MCMF solver itself. 5 unit tests were designed to verify the correctness of the assignment algorithm under controlled cost-matrix inputs. These tests focus only on algorithm-level correctness, such as feasibility, non-duplicate assignment, and behavior under shortage conditions.

s,

3.2 Stage 2: Short Simulation Validation

Stage 2 is the first validation stage that runs the MCMF logic inside a real V46-derived simulation loop. Its purpose is to verify that the MCMF path can be applied in the actual simulation environment, rather than only in unit tests.

Stage 2 uses example_v46_stage2_Yeqi.m, which is a dedicated Stage 2 copy of V46 with an event trigger.  For example, in an 8-sec simulation scenario, target 1 and target 2 are forced into simultaneous PENDING_SELECTION at timestamp=5.0 , after which the real simulation loop continues normally.

The main setup is summarized below:

- Runtime: 8 s with injection at 5 s for the standard case; an additional 30 s / 15 s case was also tested.

- Manual Event Trigger: target 1 and target 2 enter PENDING_SELECTION.

- Active trackers at injection: target 1→ [1, 2], target 2 → [3, 4].

- Cost function: unchanged from the original simulation.

- Impact on normal V46: none, because the manual event trigger is isolated in example_v46_stage2_Yeqi.m rather than the clean example_V46_Yeqi.m.

| Scenario | MCMF Assignment | Minmax Assignment | Cost Improvement |
| --- | --- | --- | --- |
| 8s/5s | T1 [6, 7], T2 [8, 9], Cost 1.2443 | T1 [7, 11], T2 [6, 8], Cost 1.3038 | 0.0595 |
| 30s/15s | T1 [6, 8], T2 [7, 9], Cost 1.0341 | T1 [8, 6], T2 [7, 12] Cost 1.0968 | 0.0626 |

Table 2: Comparison of MCMF and Minmax solutions for Interceptor Selection Conflict

Table 2 shows that the MCMF-based assignment strategy is not only algorithmically correct, but also successfully integrated into the V46 simulation framework.

4.3 Stage 3: Full Natural Simulation

Stage 3 runs the clean main script example_V46_Yeqi.m directly. No stage 3 injection file is used. Table 3 shows a summary of the natural simulation.

| MAT output | logs/wsn_data_20260408_183623.mat |
| --- | --- |
| Log output | logs/wsn_log_20260408_183623.log |
| Assignment events | 12 times, all MCMF |
| Multi-target natural assignment events | 0 |
| Successful Intercepts | 9 |
| Simulation Completion | Reached t=75s |

Table 3: Full 75-Second Simulation Output Summary

Figure 3: Simulation Under the Same Configuration as V45, with MCMF Enabled

As Figure 3 shown, MCMF is naturally invoked by the clean full simulation. However, this natural run did not produce simultaneous multi-target selection, so it does not replace the controlled conflict evidence form Stage 1&2, and Stage 3. I may need to find more test cases then.

A supplementary 15 s natural simulation case was also examined to show that MCMF can operate correctly in a naturally occurring simultaneous two-target assignment event. In this case, target 1 and target 2 naturally entered PENDING_SELECTION at the same time, without any manual injection.

MCMF:

- T1 → [17 21], cost 0.6211

- T2 → [7 11], cost 0.5849

- total 1.2060

Minmax:

- T1 → [11 17], cost 0.5997

- T2 → [7 1], cost 0.6137

- total 1.2133

Although the improvement is small, this remains a useful no-injection example in which MCMF outperformed the minmax strategy.

15 s Simulation with MCMF

5. Discussion and Future Work

A natural simultaneous multi-target PENDING_SELECTION event remains difficult to obtain consistently in the clean 75 s simulation, because the triggering process depends on multiple coupled factors, including EKF convergence, prediction stability, interception confidence, and target timing. As a next step, a supplementary relaxed-trigger simulation could be explored by loosening selected handover-trigger conditions, such as MIN_STABLE_PREDICTIONS, PREDICTION_STABILITY_THRESHOLD, and EKF_VELOCITY_CONVERGENCE_THRESHOLD. This would increase the likelihood of natural resource-conflict scenarios while preserving the MCMF objective itself.

Such an experiment would not replace the clean natural simulation but would provide an intermediate validation layer between the fully injected tests and the fully natural run.

Appendix: Source Files in ZIP

| Purpose | Files |
| --- | --- |
| Clean V46 natural simulation | example_V46_Yeqi.m |
| Stage 2 controlled simulation | example_v46_stage2_Yeqi.m, run_mcmf_stage2_short_controlled_simulation.m |
| supplementary 15sec natural simulation | example_V46_natural_conflict_Yeqi.m |
| MCMF and legacy solvers | solveInterceptorAssignments.m, solveInterceptorAssignmentMCMF.m, solveInterceptorAssignmentLegacy.m |
| Simulation adapter | buildAssignmentCostMatrixForCallingTargets.m, calculateEnhancedBid.m, getSharedTargetInfo.m, findNearbySensors.m |
| Unit Tests | test_interceptor_assignment.m, test_mcmf_simulation_adapter.m |